# Data Guide  —  v1.0

2D/3D Fluoroscopy Registration Pipeline
October 2026

# 1.  Overview

This document explains what data is needed to run each test in the pipeline, where to download it, how to organise it, and what format it must be in. The pipeline itself does NOT come with patient data. All public datasets referenced here are freely available for research use.

# 2.  DeepFluoro Dataset  —  Required for 4.3 mm Pelvis Test

## 2.1  What it is

A cadaveric pelvis CT + real intraoperative fluoroscopy dataset. 6 specimens, each with a CT volume and ~80-110 real fluoroscopy projections at different C-arm angles. Ground-truth 2D landmark positions are provided by the original Johns Hopkins researchers.

| Property | Value |
| --- | --- |
| Source | Grupp et al., Johns Hopkins University, IPCAI/IJCARS 2020 |
| License | CC BY-NC 4.0 (free for research, non-commercial) |
| Download URL | https://doi.org/10.7281/T1/IFSXNV |
| File | ipcai_2020_full_res_data.h5 (~2.5 GB HDF5) |
| Landmarks | 14 bilateral pelvis landmarks (ASIS, FH, GSN, IOF, IPS, MOF, SPS) |
| PCA spread ratio | 0.72 (well-conditioned for PnP — no collinearity problem) |

## 2.2  Where to place it

data/
└── deepfluoro/
    └── ipcai_2020_full_res_data.h5

## 2.3  Run the test

cd pipeline
python build_drr_bank_deepfluoro.py
python run_deepfluoro_multiview_test.py
# Expected: PDE = 4.3 mm

# 3.  VerSe19 Dataset  —  Required for 0.32 mm Spine Test

## 3.1  What it is

A public lumbar spine CT dataset with expert vertebra centroid annotations. We use the sub-verse010 specimen. The registration target is a synthetic DRR (not a real X-ray), which is why the 0.32 mm result is achievable — there is no synthetic-to-real domain gap when both target and bank are synthetic.

| Property | Value |
| --- | --- |
| Source | Sekuboyina et al., VerSe Challenge 2020 |
| License | See https://osf.io/nqjyw/ for terms |
| Download URL | https://osf.io/nqjyw/ |
| Specimen used | sub-verse010 |
| Landmarks | L1, L2, L3, L4, L5 vertebra centroids |
| PCA spread ratio | 0.186 — more collinear than pelvis, but acceptable for synthetic test |

## 3.2  Where to place it

data/
└── verse/
    ├── sub-verse010_ct.nii.gz
    └── sub-verse010_seg-vert_msk.nii.gz

## 3.3  Run the test

cd pipeline
python run_verse_multiview_test.py
# Expected: Mean PDE = 0.32 mm  (AP=0.52mm, LAT=0.11mm)

# 4.  NIMS Real Patient Dataset  —  Required for NIMS Pipeline Test

## 4.1  What is needed

| Item | Format | Description |
| --- | --- | --- |
| Pre-operative CT | NRRD (.nrrd) | Full 3D CT volume. Any standard CT scanner. |
| Vertebra centroid labels | 3D Slicer markup JSON (.mrk.json) | 3D positions of D12, L1-L5 vertebra centres. Annotated by radiologist/surgeon. |
| C-arm DICOM frames | Raw DICOM (Ziehm: no file extension) | 100+ intraoperative C-arm frames from surgical session. |
| AP labeled set | 3D Slicer markup JSON (.mrk.json) | 2D pixel positions of L3, L4, L5 in an AP X-ray frame. For calibration. |
| LAT labeled set | 3D Slicer markup JSON (.mrk.json) | 2D pixel positions of L2, L3, L4, L5 in a LAT X-ray frame. For calibration. |

## 4.2  Directory structure

data/
└── nims/
    ├── patient_ct.nrrd
    ├── CT_label/
    │   └── CENTROID.mrk.json
    ├── carm_frames/
    │   ├── 1
    │   ├── 2
    │   └── ...
    └── XRAY_Label/
        ├── XRAY-AP/
        │   └── AP-SET-1/
            │       └── CENTROID.mrk.json
        └── XRAY-LAT/
            └── LAT-SET-1/
                └── CENTROID.mrk.json

## 4.3  Label format  (3D Slicer markup JSON)

For CT labels (3D positions in mm, LPS coordinate system):

{
  "markups": [{
    "controlPoints": [
      {"label": "L3", "position": [x_mm, y_mm, z_mm]},
      {"label": "L4", "position": [x_mm, y_mm, z_mm]},
      {"label": "L5", "position": [x_mm, y_mm, z_mm]}
    ]
  }]
}

For X-ray labels (2D pixel positions, origin at top-left):

{
  "markups": [{
    "controlPoints": [
      {"label": "L3", "position": [u_px, v_px, 0.0]},
      {"label": "L4", "position": [u_px, v_px, 0.0]},
      {"label": "L5", "position": [u_px, v_px, 0.0]}
    ]
  }]
}

## 4.4  C-arm configuration

The pipeline uses configs/cameras/ziehm_vision_fd.json by default (SID=1050mm, pixel_spacing=0.288mm). If your C-arm is a different model, update the config file or pass a different config path. Always verify SID and pixel spacing from the actual DICOM tags before running.

## 4.5  Run the test

# Edit the USER CONFIGURATION section in the script first
cd pipeline
python run_nims_multiview_test.py
# or for auto-pedicle extraction:
python run_nims_auto_pedicle_test.py

# 5.  Training Data  —  Required for CNN Retraining

To retrain the landmark detector CNN (train_landmark_detector.py), you need the following data manifest files in data/:

data/landmark_manifest_real.csv — CSV of real X-ray images with 2D labels. Format: patient, set_name, view, img_path, labels, coords_px.

data/synthetic_drr_manifest_ap.csv — CSV of synthetic AP DRR images generated by generate_synthetic_drrs.py.

data/synthetic_drr_manifest_lat.csv — CSV of synthetic LAT DRR images.

data/converted_xray_manifest_ap.csv — CSV of CycleGAN-converted X-rays (generated by convert_xrays_to_drr.py). Optional but recommended.

These manifest files are generated automatically by the data preparation scripts. See README.md and scripts/ for instructions on building them from raw data.